Dripperline Land Application Maintenance Check List

Ref No: #

### Property details

| Address of property: |  |  |  |
| --- | --- | --- | --- |
| Owner name: |  | Maintenance contact: |  |
| Council/ Regional Authority: |  | Date: |  |

### Maintenance checklist

| Description |  |  | Action Required |
| --- | --- | --- | --- |
| Pump |  |  |  |
| Check pump for damage/ blockages to the impeller. |  |  |  |
| Check noise levels of pump in operation. If the pump is vibrating it suggests bearing may be failing. |  |  |  |
| Check pump and associated fittings for leaks. |  |  |  |
| Headworks Assembly |  |  |  |
| Inspect filter cartridge - if necessary remove and wash with pressurised water. In case of slime or carbonate build up, chemical washing with acid or chlorine is recommended, refer to manufacturer’s guidelines. |  |  |  |
| Check filters and associated fittings for leaks. |  |  |  |
| Inspect valve box (if installed) to ensure it is clean and free of any debris. |  |  |  |
| Techfilter |  |  |  |
| Inspect filter cartridge - if necessary remove and wash with pressurised water. |  |  |  |
| Replace Techfilter cartridge every 24 months. |  |  |  |
| Date of Installation: |  |  |  |
| Dripperline |  |  |  |
| While system is in operation check for leaks where ponding may occur on the surface. Use joiners if any damage to dripperline has occurred. |  |  |  |
| Uncover a section of dripperline periodically to see if the drippers are working. |  |  |  |
| Cut a section of dripperline out to visually check the inside of the tube for any blockages or damage to the emitters. This may be done once every 2 years. |  |  |  |
| Do a physical check of the boundaries around the land application area particularly on the lowest point to see if any run-off may be occurring. |  |  |  |
| If DNL’s are installed check for leaks. |  |  |  |
| Monitor and record pressures across field and compare to previous readings. |  |  |  |
| Check all dripperline fittings for leaks while in operation and that all ratchet clamps are secure. |  |  |  |
| Air Release Valve |  |  |  |
| Check air release valve for leaks while in operation and that all ratchet clamps are secure. |  |  |  |

### Flushing of system

| Flushing helps prevent dirt accumulating at the end of the pipe work. The interval between flushing depends mainly on the water quality pumped through the system. Flushing programs should start at short intervals. The interval can then be extended according to the system performance - if minimum amount of contaminant present, time between flushing can be extended; if a large amount is found, flush more often. The flushing program interval may also need to be changed at different times of the year according to irrigation frequency and water quality. |  |  |
| --- | --- | --- |
| System pressure should be kept at maximum while flushing the mainline. |  |  |
| Manually flush (clean) the filters prior to any system flushing. |  |  |
| Start by opening the flushing valve closest to the headworks assembly. Dirt will not flush instantly, out of the system, it may take several minutes before dirt starts flowing through the valve. The flushing valve should be closed, only after the majority of the dirt has been flushed out and clean water has flowed from the valve. |  |  |
| Allow for 1 second of flushing for the every meter from the headworks assembly to the flush valve. |  |  |
| Repeat this process for each zone. |  |  |

### Regulations and Legislation

Each state and council has regulations and by-laws concerning the dispersal of effluent which must be strictly complied with. This checklist is intended to be an aid to installers and end users of the land application dispersal systems and according to local, state or federal law and regulation shall take precedence over this checklist. You are accordingly advised and requested to check with your own council and state government as to their specific requirements for the installation of land application dispersal systems. Furthermore, the user of the land application dispersal systems is subject to all local conditions that prevail at the site and further subject to any appropriate expert tests including but not limited to soil tests and water tests, which may be required prior to the installation to determine the fitness for use.

In addition, other factors must be considered to determine fitness for use including but not limited to slope and landscape contours and acceptable hydraulic loading rates.

### Disclaimer

The information contained in this manual is intended to act as commentary and general information and is not intended to be advice or contain any representations that should be relied upon by the reader or recipient.  The reader or recipient should not rely upon any statement of potential performance, productivity or efficiency as these matters will depend on the individual circumstances of the reader/recipient and the reader/recipient should conduct their own independent investigations and enquiries in respect of these matters.

Any technical information is generic and should not be relied on by the reader or recipient.  Furthermore, Netafim does not warrant or make any representation regarding the information contained in this manual or the reliability of the information and the reader or recipient should not rely upon any statement or representation made in this manual.  The reader or recipient should obtain advice tailored to their particular circumstances and should not rely upon information contained in this manual for any purpose.

If the reader or recipient acquires goods or services from Netafim the relationship between Netafim and the reader/recipient will be governed by Netafim's supply terms and conditions.

The reader or recipient warrants that this manual is received subject to these conditions and that the reader or recipient will not provide this manual to any person who Netafim has not expressly authorised to receive this manual.  If, after obtaining Netafirm's express authority, this manual is provided to any other person by the reader or recipient it shall be provided to that person on the same terms set out in this disclaimer.